## Desain optimal contoh (untuk diuji di CST)

(Ini adalah contoh desain optimal berdasarkan perhitungan awal untuk antena patch rectangular pada substrat FR-4, f₀ = 10 GHz — gunakan sebagai kandidat awal;

- Frekuensi kerja: 10.0 GHz

- Substrat: FR-4, εr ≈ 4.4, h = 1.6 mm, tanδ ≈ 0.02

- Patch (top view): W = 9.129 mm, L = 6.428 mm

- Feed microstrip: lebar = 3.00 mm (perkiraan untuk ~50 Ω, verifikasi LineCalc)

- Inset feed: x = 2.35 mm (kedalaman inset dari tepi patch)

- Ground plane (prototipe): 30 × 30 mm

- Ketebalan tembaga: t_cu = 35 µm (0.035 mm) atau 17 µm sesuai pabrik PCB

- Target performa: S11 < −10 dB di 10 GHz; Gain ≈ 5–7 dBi; Bandwidth target ±250–500 MHz

Catatan: jika hasil optimasi DNN/GA memberi dimensi lain, langsung gunakan angka optimumnya — lalu jalankan verifikasi full-wave di CST.

## Script Python (jalankan di Colab / Jupyter) — buat slide PNG

Salin seluruh kode berikut ke satu sel Jupyter/Colab dan jalankan. File hasil: optimal_antenna_slide.png akan muncul di working directory (di Colab bisa diunduh dari panel Files).

# Script: buat_slide_antenna.py

# Men-generate slide PNG 2D layout antena mikrostrip (top view) beserta parameter.

# Jalankan di Jupyter Notebook atau Google Colab.

import matplotlib.pyplot as plt

import matplotlib.patches as patches

import os

out_png = "optimal_antenna_slide.png"

# -- Parameter contoh (ganti dengan hasil optimasi Anda jika ada) --

params = {

"Frekuensi kerja": "10.0 GHz",

"Substrat": "FR-4 (εr ≈ 4.4), h = 1.6 mm",

"W_patch": 9.129,    # mm

"L_patch": 6.428,    # mm

"Feed width": 3.0,   # mm

"Inset (x)": 2.35,   # mm

"Ground plane": "30 × 30 mm",

"Target": "S11 < -10 dB, Gain ~5–7 dBi, BW ±250–500 MHz"

}

fig_w, fig_h = 12, 8  # inches

dpi = 150

fig, ax = plt.subplots(figsize=(fig_w, fig_h), dpi=dpi)

ax.set_xlim(-20, 20)

ax.set_ylim(-12, 12)

ax.set_aspect('equal')

ax.axis('off')

ground_w = 30.0

ground_h = 30.0

ground_rect = patches.Rectangle((-ground_w/2, -ground_h/2), ground_w, ground_h,

linewidth=1.2, edgecolor='k', facecolor='#f7fbff')

ax.add_patch(ground_rect)

W = params["W_patch"]

L = params["L_patch"]

patch_rect = patches.Rectangle((-W/2, -L/2), W, L, linewidth=2.0, edgecolor='#1f77b4', facecolor='#9ecae1')

ax.add_patch(patch_rect)

feed_w = params["Feed width"]

inset_x = params["Inset (x)"]

feed_start_x = -ground_w/2

feed_end_x = -W/2 + inset_x

feed_rect = patches.Rectangle((feed_start_x, -feed_w/2), feed_end_x - feed_start_x, feed_w,

linewidth=1.0, edgecolor='k', facecolor='#bdbdbd')

ax.add_patch(feed_rect)

# Dimensions arrows and annotations

def draw_dim(x1, y1, x2, y2, text, offset=0.8):

ax.annotate('', xy=(x1,y1), xytext=(x2,y2), arrowprops=dict(arrowstyle='<->', lw=1.0))

mx, my = (x1+x2)/2, (y1+y2)/2

ax.text(mx, my+offset, text, ha='center', va='bottom', fontsize=10, bbox=dict(boxstyle="round,pad=0.2", fc="white", ec="none", alpha=0.9))

draw_dim(-W/2, L/2+1.0, W/2, L/2+1.0, f"W = {W:.3f} mm", offset=0.2)

draw_dim(W/2+1.0, -L/2, W/2+1.0, L/2, f"L = {L:.3f} mm", offset=0.2)

draw_dim(feed_start_x, -feed_w/2-1.2, feed_start_x, feed_w/2+1.2, f"Feed W ≈ {feed_w:.2f} mm", offset=0.1)

ax.annotate('', xy=( -W/2, -L/2 - 3.0), xytext=( -W/2 + inset_x, -L/2 - 3.0), arrowprops=dict(arrowstyle='<->', lw=1.0))

ax.text(-W/2 + inset_x/2, -L/2 - 3.5, f"Inset x = {inset_x:.2f} mm", ha='center', va='bottom', fontsize=10, bbox=dict(boxstyle="round,pad=0.2", fc="white", ec="none", alpha=0.9))

ax.text(0, 10.5, "Desain Antena Mikrostrip — Kandidat Optimal (Contoh)", ha='center', va='center', fontsize=14, fontweight='bold')

param_text = "\n".join([f"{k}: {v}" for k,v in params.items()])

props = dict(boxstyle='round', facecolor='white', alpha=0.95)

ax.text(15, 4, "Parameter Desain (Contoh):\n\n" + param_text, fontsize=10, va='top', ha='left', bbox=props)

ax.text(0, -10.5, "CATATAN: Desain ini bersifat ilustratif (hasil awal). Verifikasikan kembali di CST/HFSS dan fabrikasi untuk validasi.", ha='center', va='center', fontsize=9, color='gray')

plt.savefig(out_png, bbox_inches='tight')

plt.close(fig)

print("Selesai. File:", out_png)

Jika menjalankan di Google Colab: buat file baru, tempel kode di atas, jalankan; hasil PNG akan muncul di panel file (kiri) — klik kanan → Download.